IOP Test Plan

ADC Filter Driver

| Project Name | IOP Extension Package |
| --- | --- |
| Project ID |  |
| Document ID | IOP_ADCFLTR_TEST_CASES.doc |
| Version | 1.0 |
| Date | December 8th, 2008 |
| Status | Waiting for approval |
| iCAP Classification | Freescale Internal Use Only |

| Approval list |  |  |
| --- | --- | --- |
| Role | Name | Date |
| SSE Applications Engineer | Oscar Luna |  |
| SSE Applications Engineer | Ioseph Martínez |  |

Version Tracking

| Date | Version | Comment | Author |
| --- | --- | --- | --- |
| December 8th, 2008 | 1.0 | Initial draft | R01160 |

Table of Contents

1.	Scope of this document	1

2.	Acronyms and definitions	1

3.	General test suite conditions	1

3.1.	Environment prerequisites (installation and network)	1

3.2.	Test pre-conditions (data created before testing)	1

3.3.	Other sources and prerequisites information	2

3.4.	Test Cases enablement	2

3.5.	Tables & Diagrams	2

# Scope of this document

This document describes in detail the test plan description of the ADC Filter driver for 5510 family using the Zen core (Z0) to meet customer requirements. For each test of a particular requirement, an expected state/response is detailed. The result of expected state/response is the acceptance criteria.

Reference Documents:

MPC5510_Reference_Manual_Rev0.pdf

| Type of Test Case | Test Case ID |
| --- | --- |
| Customer | ADCFLTR_TCxxx |

Table 1.0. Test Case Types

# Acronyms and definitions

| Acronyms | Definitions |
| --- | --- |
| IOP | Input/Output Processor |
| Z0 | Identical Second core of the MPC5510 |
| ADCFLTR | ADC Filter |

# General test suite conditions

The AdcFltr driver will be tested using the Freescale’s 5510 microcontroller family according to Customer ADC Filter requirements. A CodeWarrior v2.3 will be used for testing.

## Environment prerequisites (installation and network)

- MPC5516 Evaluation board.

- Lauterbach Debugger with Trace32 Software preinstalled or P&E Multilink.

- CodeWarrior v2.3.

- 1 Oscilloscope probe, ONE 4-Ch Digital oscilloscope.

- Triple Output Power Supply.

## Test pre-conditions (data created before testing)

- Create a complete test plan for the ADC Filter driver covering the most important features of the driver.

2)   Have a complete set of test cases definition to be used by the final tester.

- Test cases source code.

- Test case document to report final results of each test.

## Other sources and prerequisites information

NA

## Test Cases enablement

Each Test case MUST be enabled when no other test case is activated. The enablement of two test cases at a time WILL cause some errors or malfunctions in the operation of the ADC Filter driver.

To activate a test case in the source code, user shall go directly to code file named “AdcFltr_Cfg.h” and switch the current state from the desired Test case to be activated from “OFF” to “ON”.

## Tables & Diagrams

4. Test Cases

| TC ID | ADCFLTR_TC001 |  |  |  | Covered requirement(s) |  | Test Time |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  | 10 min |
| Objectives |  | Test ADC Filter Error layer using wrong configuration parameters and null pointer structures |  |  |  |  |  |
| Preconditions |  |  | Inputs | Procedure |  | Expected results |  |
| 1) Det error layer shall be available within the context of this driver. |  |  | 1) “vfnAdcFltr_Init” function with NULL pointer. | 1) Enable Development Error Detection switch before compilation. Located at AdcFltr_Cfg.h file. “ADC_FLTR_ERROR_DETECT ON” 2) Call vfnAdcFltr_Init function with Null pointer as the parameter. |  | 1) vfnAdc_Filter_Report_Error function with shall report the follow parameters: ADC_FLTR_CALLBACK_PTR_ID (0x01), ADC_FLTR_E_FNC_CALLBACK_UNINIT (0x14). 2) Check variables “Adc_Filter_ApiId” and “Adc_Filter _ErrorId” 3) Initialization of the ADC Filter should be skipped. |  |
| Pass criteria |  |  |  |  |  |  |  |

| TC ID | Project Identifier |  |  | Test Type |  | Test Time |
| --- | --- | --- | --- | --- | --- | --- |
| ADCFLTR_TC003 | G013 - IOP Extension Package |  |  | White Box |  | 15 min |
| Objectives |  | Validate the sampling range allowed by the ADC Filter driver. Filter driver allows only sample depth ranges such as: 2,4,8,16,32 & 64 |  |  |  |  |
| Preconditions |  | Inputs | Procedure |  | Expected results |  |
| Det error layer shall be available within the context of this driver. |  | 1) “vfnAdcFltr_Init” function with initialized pointer. 2) SAMPLE_AVERAGE_VALUE definition initialized with an invalid range value. | 1) Enable Development Error Detection switch before compilation. Located at AdcFltr_Cfg.h file. “ADC_FLTR_ERROR_DETECT ON” 2) Call vfnAdcFltr_Init function with “AdcFltrCntl_t” initialized pointer as the parameter. 3) Debug throughout vfnAdcFltr_Init function until Filter error catch the sample range error. |  | 1) vfnAdc_Filter_Report_Error function with shall report the follow parameters: ADC_FLTR_INIT_ID (0x00), ADC_FLTR_E_AVERAGE (0x13). 2) Check variables “Adc_Filter_ApiId” and “Adc_Filter _ErrorId” 3) Initialization of the ADC Filter should be skipped. |  |
| Pass criteria |  |  |  |  |  |  |

| TC ID | Project Identifier |  |  | Test Type |  | Test Time |
| --- | --- | --- | --- | --- | --- | --- |
| ADCFLTR_TC002 | G013 - IOP Extension Package |  |  | White Box |  | 10 min |
| Objectives |  | Validate the correct initialization of the ADC Filter driver by passing the correct parameters through the function called “vfnAdcFltr_Init” |  |  |  |  |
| Preconditions |  | Inputs | Procedure |  | Expected results |  |
| Det error layer shall be available within the context of this driver. |  | 1) “vfnAdcFltr_Init” function with initialized pointer. | 1) Enable Development Error Detection switch before compilation. Located at AdcFltr_Cfg.h file. “ADC_FLTR_ERROR_DETECT ON” 2) Call vfnAdcFltr_Init function with “AdcFltrCntl_t” initialized pointer as the parameter. |  | 1)vfnAdc_Filter_Report_Error function shall no report any type of error. 2) The Error Layer shall let the vfnAdcFltr_Init driver continue with the configuration process. 3) Initialization of the ADC Filter shall be correctly performed. |  |
| Pass criteria |  |  |  |  |  |  |
| Comments: |  |  |  |  |  |  |

| TC ID | Project Identifier |  |  | Test Type |  | Test Time |
| --- | --- | --- | --- | --- | --- | --- |
| ADCFLTR_TC004 | G013 - IOP Extension Package |  |  | White Box |  | 20 min |
| Objectives |  | Validate the initialization of 4 ADC channels as inputs and validate each pin to be configured as ADC input mode rather than GPIO mode |  |  |  |  |
| Preconditions |  | Inputs | Procedure |  | Expected results |  |
| Det error layer shall be available within the context of this driver. |  | 1) “au8Adc_Port_Pins” array located at “Adc_Cfg.c” configured with four channels 2) Define named “MAX_ADC_CHANNELS” configured with value number 4. | 1) Enable Development Error Detection switch before compilation. Located at AdcFltr_Cfg.h file. “ADC_FLTR_ERROR_DETECT ON” 2) Debbug using CW 6.3 until function “vfnAdc_Init_Port_Pins” be reached. 3) Inside the “vfnAdc_Init_Port_Pins” function go inside the for loop and debug each iteration step by step. |  | - ADC Channels 0 , 1, 2 and 3 shall be configured as inputs and their ADC functionality enabled. - SIU.PCR[0..3].R shall contain the value of 0x0500 indicating that the four pins were configured correctly. |  |
| Pass criteria |  |  |  |  |  |  |

| TC ID | Project Identifier |  |  | Test Type |  | Test Time |
| --- | --- | --- | --- | --- | --- | --- |
| ADCFLTR_TC005 | G013 - IOP Extension Package |  |  | Black Box |  | 30 min |
| Objectives |  | Average 4 different ADC Channels with a depth of 8 samples. Channels to be averaged are 0, 1 ,2 and 3. |  |  |  |  |
| Preconditions |  | Inputs | Procedure |  | Expected results |  |
| - Det error layer shall be available within the context of this driver. - Triple Power Supply |  | 1) ADC Channel 0 (AN0 – Pin 0) with 2.60 volts. 2) ADC Channel 1 (AN1 – Pin 1) with 1.68 volts. 3) ADC Channel 2 (AN2 – Pin 2) with 2.60 volts. 4) ADC Channel 3 (AN3 – Pin 3) with 2.60 volts. | 1) Enable Development Error Detection switch before compilation. Located at AdcFltr_Cfg.h file. “ADC_FLTR_ERROR_DETECT ON” 2) Call vfnAdcFltr_Init function with “AdcFltrCntl_t” initialized pointer as the parameter. 3) Set a breakpoint at function named “ vfnFilter_1_End” 4) Run thru the “ vfnFilter_1_End” function. |  | - The follow variables named shall contains the next filtered values or NEAR values : u16Total[0] = 0x085C u16Total[1] = 0x05DF u16Total[2] = 0x085C u16Total[3] = 0x085C |  |
| Pass criteria |  |  |  |  |  |  |
| Comments: |  |  |  |  |  |  |

| TC ID | Project Identifier |  |  | Test Type |  | Test Time |
| --- | --- | --- | --- | --- | --- | --- |
| ADCFLTR_TC006 | G013 - IOP Extension Package |  |  | Black Box |  | 30 min |
| Objectives |  | Average 2 different ADC Channels with a depth of 4 samples. Channels to be averaged are 0 and 1. |  |  |  |  |
| Preconditions |  | Inputs | Procedure |  | Expected results |  |
| - Det error layer shall be available within the context of this driver. - Triple Power Supply |  | 1) ADC Channel 0 (AN0 – Pin 0) with 1.30 volts. 2) ADC Channel 1 (AN1 – Pin 1) with 3.24 volts. | 1) Enable Development Error Detection switch before compilation. Located at AdcFltr_Cfg.h file. “ADC_FLTR_ERROR_DETECT ON” 2) Call vfnAdcFltr_Init function with “AdcFltrCntl_t” initialized pointer as the parameter. 3) Set a breakpoint at function named “ vfnFilter_1_End” 4) Run thru the “ vfnFilter_1_End” function. |  | - The follow variables named shall contains the next filtered values or NEAR values : u16Total[0] = 0x0442 u16Total[1] = 0x0A5D |  |
| Pass criteria |  |  |  |  |  |  |

| TC ID | Project Identifier |  |  | Test Type |  | Test Time |
| --- | --- | --- | --- | --- | --- | --- |
| ADCFLTR_TC007 | G013 - IOP Extension Package |  |  | Black Box |  | 30 min |
| Objectives |  | Average 6 different ADC Channels with a depth of 16 samples. Channels to be averaged are 0 ,1, 2, 3, 4 and 5. |  |  |  |  |
| Preconditions |  | Inputs | Procedure |  | Expected results |  |
| - Det error layer shall be available within the context of this driver. - Triple Power Supply |  | 1) ADC Channel 0 (AN0 – Pin 0) with 0.50 volts. 2) ADC Channel 1 (AN1 – Pin 1) with 1.50 volts. 3) ADC Channel 0 (AN2 – Pin 2) with 2.30 volts. 4) ADC Channel 1 (AN3 – Pin 3) with 0.50 volts. 5) ADC Channel 0 (AN4 – Pin 4) with 1.50 volts. 4) ADC Channel 1 (AN5 – Pin 5) with 2.30 volts. | 1) Enable Development Error Detection switch before compilation. Located at AdcFltr_Cfg.h file. “ADC_FLTR_ERROR_DETECT ON” 2) Call vfnAdcFltr_Init function with “AdcFltrCntl_t” initialized pointer as the parameter. 3) Set a breakpoint at function named “ vfnFilter_1_End” 4) Run thru the “ vfnFilter_1_End” function. |  | - The follow variables named shall contains the next filtered values or NEAR values : u16Total[0] = 0x0195 u16Total[1] = 0x04CC u16Total[2] = 0x075A u16Total[3] = 0x0195 u16Total[4] = 0x04CC u16Total[5] = 0x075A |  |
| Pass criteria |  |  |  |  |  |  |

| TC ID | Project Identifier |  |  | Test Type |  | Test Time |
| --- | --- | --- | --- | --- | --- | --- |
| ADCFLTR_TC008 | G013 - IOP Extension Package |  |  | Black Box |  | 30 min |
| Objectives |  | Average 4 different ADC Channels with a depth of 32 samples. Channels to be averaged are 0 ,1, 2 and 3. |  |  |  |  |
| Preconditions |  | Inputs | Procedure |  | Expected results |  |
| - Det error layer shall be available within the context of this driver. - Triple Power Supply |  | 1) ADC Channel 0 (AN0 – Pin 0) with 0.50 volts. 2) ADC Channel 1 (AN1 – Pin 1) with 1.50 volts. 3) ADC Channel 0 (AN2 – Pin 2) with 2.30 volts. 4) ADC Channel 1 (AN3 – Pin 3) with 0.50 volts. | 1) Enable Development Error Detection switch before compilation. Located at AdcFltr_Cfg.h file. “ADC_FLTR_ERROR_DETECT ON” 2) Call vfnAdcFltr_Init function with “AdcFltrCntl_t” initialized pointer as the parameter. 3) Set a breakpoint at function named “ vfnFilter_1_End” 4) Run thru the “ vfnFilter_1_End” function. |  | - The follow variables named shall contains the next filtered values or NEAR values : u16Total[0] = 0x0195 u16Total[1] = 0x04CC u16Total[2] = 0x075A u16Total[3] = 0x0195 |  |
| Pass criteria |  |  |  |  |  |  |

| TC ID | Project Identifier |  |  | Test Type |  | Test Time |
| --- | --- | --- | --- | --- | --- | --- |
| ADCFLTR_TC009 | G013 - IOP Extension Package |  |  | Black Box |  | 10 min |
| Objectives |  | Validate the Filter End function by pointing to a different customized function. |  |  |  |  |
| Preconditions |  | Inputs | Procedure |  | Expected results |  |
| - Det error layer shall be available within the context of this driver. - Triple Power Supply |  | 1) ADC Channel 0 (AN0 – Pin 0) with 0.50 volts. 2) ADC Channel 1 (AN1 – Pin 1) with 1.50 volts. 3) ADC Channel 0 (AN2 – Pin 2) with 2.30 volts. 4) ADC Channel 1 (AN3 – Pin 3) with 0.50 volts. | 1) Enable Development Error Detection switch before compilation. Located at AdcFltr_Cfg.h file. “ADC_FLTR_ERROR_DETECT ON” 2) Call vfnAdcFltr_Init function with “AdcFltrCntl_t” initialized pointer as the parameter. 3) Set a breakpoint at function named “vfnFilter_1_End_TC_8” 4) Run thru the “vfnFilter_1_End_TC_8” function. |  | - Adc Filter Driver shall jump into “vfnFilter_1_End_TC_8” once the filter finishes to average 8 samples. - The follow variables named shall contains the next filtered values or NEAR values : u16Total[0] = 0x0195 u16Total[1] = 0x04CC u16Total[2] = 0x075A u16Total[3] = 0x0195 |  |
| Pass criteria |  |  |  |  |  |  |