| TM |  |
| --- | --- |
|  | February 21, 2008 |

# MX27 WinCE 6.0 Performance Analysis

Abstract:

Performance Analysis for MX27 WinCE 6.0 for Decoder

Keywords:

WinCE, Direct Show, MX27

Revision History

| Version |  | Date |  | Author |  | Change Description |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 1.0 |  | 15-Feb-2008 |  | Lauren Post |  | Initial Draft |  |
|  | 1.1 |  | 20-Feb-2008 |  | Lauren Post |  | Updated with results |  |
|  | 1.2 |  | 28-Feb-2008 |  | Lauren Post |  | Updated with Dropped Frame results |  |

Table of Contents

1	Introduction	4

1.1	Purpose	4

1.2	Scope	4

Audience Description	4

1.3	References	4

2	VPU Decoder Performance	5

2.1	Visual Testing	5

2.2	Performance Results	6

2.2.1	Breakdown	6

2.2.2	Frame Rate Achieved	6

2.3	Dropped Frame Support	7

3	Performance Analysis	9

3.1	Conclusion	9

4	BSP Issues	11

# Introduction

## Purpose

The goal of this document is to provide a performance analysis of WinCE 6.0 MX27 VPU Decoder

## Scope

This document will describe just the MX27 VPU Decoder only without audio because the MX27 WinCE audio codecs are not optimized (Linux ARM9 optimizations will not build on WinCE)

## Audience Description

This document is intended for engineers familiar with the Freescale multimedia codecs as well as the WinCE BSP platforms.

This section shall identify the overall purpose of this document.

This section shall identify the audience of the document .If these people don’t constitute the distribution list of this document, precise this last.

## References

- WinCE MX27 VPU Filter release notes

This section shall list all documents referenced in this document. The references shall be numbered sequentially in brackets [Id] and list the document number and the document title. The document version or date and specific pages, sections and/or paragraphs should be noted if applicable.

# VPU Decoder Performance

Performance was assessed through the following analysis

- Visual testing

- Timing measurements

The current VPU implement has one remaining memcpy implemented which is the memcpy from the output of the Post Processor to the Output buffer provide by the Video Renderer.  Currently the VPU driver does not provide a consistent way to assess for physically contiguous output buffers to use with Post Processor.  Without out this, using the output buffer provided by the video renderer (from InitializeOutputSample) if not physically contiguous will cause segmentation faults using the VPU decoder.

All performance analysis was done with Dropped Frame support disabled.

## Visual Testing

Visual testing was done on D1, VGA and QVGA content both 15fps and 30fps with and without audio.   Testing with audio was done to test for audio breaks however AAC and MP3 audio on MX27 are not optimized so content with audio was not used for any performance measurements.  The content tested was the Zune content which was same clip with sync points.  The following are the results with audio

- D1 resolution lowest frame rate at 12fps, lowest bit rate at 500kbps had broken audio and video was behind.

- VGA resolution 15fps had smooth audio but video was too slow and did not stay in sync

- QVGA resolution 30fps 1500kbps played well and was synchronized with audio.

For timing measurements and remaining visual testing, the audio track was removed to assess VPU performance only.   Performance testing was done only for 3 minutes of each clip through motorcycle scene.  The following are the results

- Tearing occurred on all resolutions VGA or higher H.264 and MPEG4 regardless of frame rate or bitrate.    Lower frame rates and bit rates produced less tearing.

- For VGA clips without audio the video performance improved but was still unacceptable with tearing and speed playing past the end of the video.

- For D1, only 12fps 500kbps played acceptable in some cases– any higher frame rates or higher bitrates the video was too slow. For clips without audio, the video performance was still very poor where it either hung or continued to play after slider was complete.

## Performance Results

To discover the highest frame rate the MX27 WinCE 6.0 VPU Filter could achieve, measurements were done on the same clip encoded with various frame rates, bitrates and resolutions.  The audio track was removed and only the first 3 minutes were kept.   The total time in microseconds was gathered for each of these clips played with CEPlayer and/or standalone testbench.  Along with this sub times for VPU decode time, Post Processing time and memcpy time were saved with percentage of each provided in spreadsheet linked below

### Breakdown

Memcpy was consistent for all bitrates but varied for resolution and frame rates.

- For QVGA, memcpy was only 1-2% for 15fps, and 4-5% for 30fps.

- For VGA and D1, memcpy was 7-9% for 15fps and 10-12% for 30fps.

Post Processing was consistent for all bitrates but varied for resolution and frame rates

- For QVGA, Post Processing time was 4-5% for 15fps and 8-9% for 30fps

- For VGA, Post Processing time was 16% for 15fps and 20-25% for 30fps for MPEG4 and 10% for H.264

- For D1, Post Processing

### Frame Rate Achieved

This calculation used the total number of frames divided by the total time to calculate the number of frames per second.   For a 30fps, if the achieved rate was 20fps then the performance was too slow.  The dropped frame algorithm was not enabled so video played past the end of the clip.

#### H.264

Note that all D1 content hung in CEPlayer but played in standalone testbench (problem is in dshow pipeline getting overloaded). Estimates were made based on testbench performance

All video VGA or higher had tearing in motion with some frames fully striped.  For D1 15fps or higher, video was jerky and/or too slow.

- QVGA all  clips ranging from 15fps to 30fps played at target frame rates

- VGA 15fps played at 15fps but had tearing on motion scenes

- VGA 30fps played at 20fps but had tearing on motion scenes and in some cases striped video on entire frame.

- D1 12fps with standalone testbench with raw content, measurements were done playing as fast as possible and it played 15-17fps.

- D1 15fps with standalone testbench with raw content, measurements were done playing as fast as possible and it played 14-17fps

- D1 30fps with standalone testbench with raw content, measurements were done playing as fast as possible and it played 9-15fps

#### MPEG4

All content was played with CEPlayer with no problems other than slower video for all frame rates of 30fps and VGA or higher resolutions.   Testbench measurements were also taken to evaluate CEPlayer overhead.   All content with VGA resolution or higher had tearing during motion – sometimes generating full striped frames.

- QVGA all clips ranging from 15 to 30fps played at target frame rates

- VGA 15fps played at 15fps

- VGA 30fps played at 20fps

- D1 12fps played at 12fps plays well but with some tearing

- D1 15fps played at 15fps – video was jerky

- D1 30fps played at 20fps  - video was too slow

Note: Using MPEG4 measurements CEPlayer showed about 10-30% overhead of testbench on running D1 content.  This overhead was used to calculate CEPlayer achieved frame rates for H.264 D1 using test bench results.

## Dropped Frame Support

There are two types of dropped frame support.  Aggressive frame dropping requires Drop before decode which will drop multiple frames at once.   Minimal frame dropping requires Drop at Render which drops after decode but before Post Processing and the final memcpy to the output buffer.

#### Drop at Render

Because the overhead of flipping includes Post Processing and memcpy – doing a dropped frame support at render is optimal to avoid aggressive frame dropping and provide better smoothness control in dropping such as never dropping two frames in a row that occurs with drop before algorithms.  For higher frame rates both Drop Before Decode and Drop at Render can bring better audio video synchronization since Drop at Render is not sufficient.

The advantage to Drop At Render is that it has all frames to use as reference for future decoding – only the final algorithm of post processing and memcpy is avoided for performance.  If Drop at Render is sufficient it provides smoother video as it never drops two frames in a row.  The disadvantage to this algorithm is that it still uses VPU to decode frames which are not displayed (but still used as reference for future decoding)

#### Drop Before Decode

Drop Before Decode is more aggressive frame dropping that provides a complete solution to a/v sync.  Drop At Render still does the decode which takes time but if the sample time is too far delayed then even decode time can be prevented with Drop Before Decode. The disadvantage to this algorithm is that if drop before decode occurs it requires waiting for a key frame before resuming decode.  This means several frames in series will be discarded.  This is required to prevent garbage video from appearing.

To use in sync with Drop At Render, the Drop Before Decode calculates the key frame interval and tries not to drop until it is closer to the next key frame interval.

#### Results with Frame Dropping

Statistics were added to Stop Streaming so that number of frames dropped in total, drop at render and drop before decode were listed.  Together a calculation was used to detect the achieved frame rate with frame dropping.  Below are the following results.    All tests were done with CEPlayer in a direct show framework with FSL Mp4Parser providing valid sample start times for a/v sync.  All audio was smooth with no break.  Without frame dropping VGA or higher had audio breaks.

H.264 Results

- For VGA, 15fps, with unoptimized audio – achieved frame rate was 7 fps.

- For VGA 30fps with unoptimized audio – achieved frame rate was 9 fps.

- For VGA 15fps without audio, achieved frame rate was 8-9 fps.

- For D1 15fps with unoptimized audio, achieved frame rate was 7 fps.

- For D1 30fps with unoptimized audio, achieved frame rate was 8 fps.

- For D1 15fps without audio, achieved frame rate was 8-9 fps.

MPEG4 Results

- For VGA, 15fps, with unoptimized audio – achieved frame rate was 12-14 fps.

- For VGA 30fps with unoptimized audio – achieved frame rate was 10-11 fps.

- For VGA 15fps without audio, achieved frame rate was 11-14 fps.

- For D1 15fps with unoptimized audio, achieved frame rate was 11-14 fps.

- For D1 30fps with unoptimized audio, achieved frame rate was 9-10 fps.

- For D1 15fps without audio, achieved frame rate was 11-14 fps.

When optimized audio codecs are provided for MX27 WinCE 6.0 platform then dropped frame results should be redone to see if there is improvement.

# Performance Analysis

The MX27 WinCE 6.0 VPU filter is slower than the VPU on MX32.  This is for the following reasons:

- A single memcpy is still required because of BSP limitations in providing physical contiguous memory for output buffer.

The current VPU implementation has one memcpy from the output of the PostProcess to the Video Renderer Sample.  The VPU Filter gets the output buffer by requesting a sample from the downstream Video Renderer through the InitializeOutputSample.   This buffer is then passed to the VPU driver to get the physical address with the vpu_GetPhysAddrFromVirtAddr.  This call will return success if memory is physical or fail if it is not.  If this fails then memcpy has to be used.  Current VPU driver always returns success which causes segmentation faults because the output buffer is not physical memory (this is being investigated by BSP team)

- MX32 achieved optimal performance (as well as MX31 SW video codecs) by taking advantage of shared memory with IPU through overlay support.  On MX27 , overlay support is not enabled in BSP because of HW issues with LCD not supporting key colors correctly.  If enabled it would cause menus on applications to get corrupted.   If overlay support were available it would still require changes to allocate physical memory.   If this occurred, the overlay support provided would only allow a smaller overlay solution than the one provided in MX32 explained more in the next reason.

Without overlay support, the output buffers for the VPU filter which come from the Default Video Renderer will never be physically contiguous since it is dependent on the allocator of the video renderer which currently does not allocate physical memory on MX27.  Overlay solution is the only way to guarantee a consistent method of getting physically contiguous memory for Post Processor Output and remove the final memcpy.   This would only give 5-10% improvement at best for VGA or higher resolution.

- VPU rotation is required to provide input to Post Processing.  This increases the VPU total decode time by up to 6% and also limits any possible overlay solution to 1 or 2 buffers instead of all shared memory as MX32 VPU did with IPU. On MX32, the IPU did the rotation and color conversion while on MX27, the VPU does the rotation at the end of the decode and the Post Processing does the color conversion to the output buffer.

Without VPU rotation software overhead would be required to flip image.  Without VPU rotation enabled, image displays inverted.

- CEPlayer has about 10-30% overhead compared to Testbench

## Conclusion

VPU Filter in a Direct Show Framework for use case in CE Player is limited in performance for the following reasons

- Post Processing is part of VPU Filter instead of IPU block which requires VPU rotation as input increasing total VPU decode time.

- Memcpy from Post Processor to Output which can not be fixed with similar overlay solution from MX32

- The WinCE Framework limits optimal performance solution.   It is possible that a specific application could be written that rendered its own video and could avoid memcpy but it would only improve performance by up to 10% which would still not provide VGA or D1 at 30fps.

- Testing was done on both CE Player with container content and with standalone testbench with raw elementary content. Standalone test bench showed approximately 10-30% improvement for D1 over Ceplayer.

- Dropped frame support is required to provide a/v sync for VGA or higher resolutions but 30fps will only get about 30% frame rate with dropped frame support enabled.

Performance Overhead

- Memcpy – 5-10%

- VPU Rotation up to 6%

- CEPlayer – 10-30%

# BSP Issues

There were several BSP issues found that affected both performance analysis and performance improvements

- VPU Driver API vpu_GetPhysAddrFromVirtAddr always returns success.  It is supposed to return failure if the memory is not physically contiguous.  If this worked then memcpy could be avoided in some cases (However in Direct Show Framework when Default Video renderer is used – it will always return failure for physical memory so will not solve memcpy performance issue if this is fixed)

- Video Tearing caused by lack of timing to PP to graphics driver.  This is work that is known but has not been allocated with resources.   There is a known HW issue but a software workaround can be provided for sync method between Post Processor and LCD.

- Concerns about memory fragmentation.  On MX32 using AllocPhysMem for internal VPU memory caused memory fragmentation over several hours.  This was solved by creating dummy overlay surfaces to access constant physical memory in IPU.  There is no other solution at this point both on WinCE 5.0 and 6.0.   MX27 VPU uses AlllocPhysMem through VPU driver but will have the same problem because AllocPhysMem should only be used at boot time and not dynamically through filter usage.  One solution is to make VPU driver allocate its memory at boot time for worst case and manage it’s memory – this would be significant work in the VPU driver but it is the only way to fix a memory leak caused by the memory fragmentation.