Procedures to flash the original demo image onto the i.MXS Development Kit

Software Used

- ARM Developer Suite V1.2

Hardware Used

- ARM Multi-ICE debug unit

Procedures:

1)  Place the 8 MB bin file “FSL_MXSdevkitImage_Flash8MB_RTM.bin”, the

“Flash Loader J3 x16.new.axf” AXF, and the “AXD_MXS_flash_demo.txt” script in the same folder location.

2)  Open the script file (AXD_MXS_flash_demo.txt file) and change the two lines towards the end to have the correct paths.

3)  Set board to bootstrap mode (S11 switches 1 and 2 ON).

4)  Connect the board via JTAG using multi-iCE to the AXD debugger.  Do not use an init script to initialize board at this time.  The “AXD_MXS_flash_demo.txt” script has this initialization already.

5)  Type the following command at the AXD debug prompt:

obey "<path to folder location>\AXD_MXS_flash_demo.txt" and then enter.  Make sure to use the "" for the entire path or the obey will not work

6)  Wait until the AXD console says "Finish" AND THEN a second line prints "OK".  Then the Debug log will say "program terminated normally."  AT THIS POINT, the board has been flashed and you can unplug.

7)  Switch the board S11 switches 1 and 2 to OFF and power up the board.

You should see the Freescale logo appear after the green bootloader and then a gui with 4 applications, one of which is a FSL logo.