Note:  ‘*’ is used to indicate the default setting.

| # Jumper Settings |  | Function |
| --- | --- | --- |
| JP1 & 3 | ON/OFF* | CAN Transceiver Mode |
| JP2 & 4 | ON*/OFF | CAN Termination |
| JP5 | *1-2/2-3 | +2.5V/+3.3V Voltage Selection |
| JP7 | *ON/OFF | CPU Core Voltage Supply |
| JP8 | *1-2/2-3 | 64Mbit Flash Boot dBug/System |
| JP9 | *ON/OFF | CS0 to Flash Memory |
| JP10 | *1-2/2-3 | FPGA Clock Source Selection |
| JP11 | ON*/OFF | +3.3V CPU supply current measurement |
| JP12 | ON*/OFF | +1.5V CPU supply current measurement |
| JP13 | ON*/OFF | +2.5V CPU supply current measurement |
| JP14 | *1-2/2-3 | CPU, FPGA, PCI & SRAM Clock Source Selection |
| JP15 | *ON/OFF | DTIN0 LED Enable/Disable |
| JP16 | *ON/OFF | DTIN1 LED Enable/Disable |
| JP17 | *ON/OFF | DTIN2 LED Enable/Disable |
| JP18 | *ON/OFF | DTIN3 LED Enable/Disable |
| JP19 | *ON/OFF | DTOUT0 LED Enable/Disable |
| JP20 | *ON/OFF | DTOUT1 LED Enable/Disable |
| JP21 | *ON/OFF | DTOUT2 LED Enable/Disable |
| JP22 | *ON/OFF | DTOUT3 LED Enable/Disable |
| JP23 | *ON/OFF | TXD Isolation for the iRDA controller |
| JP24 | *ON/OFF | RXD Isolation for the iRDA controller |
| JP25 | *1-2/2-3 | iRDA controller Mode Selection |
| JP26 | *ON/OFF | UART0 /RTS Isolation |
| JP27 | *ON/OFF | UART0 /CTS Isolation |
| JP28 | *ON/OFF | UART1 /RTS Isolation |
| JP29 | *ON/OFF | UART1 /CTS Isolation |
| JP30 | *1-2/2-3 | BDM/JTAG selection |
| SW1 | All open | Ethernet 0 Physical Address (1) |
| SW2 | SW2-1 closed | Ethernet 1 Physical Address (3) |
| SW4 | All open/off | Reset Configuration of the CPU |