=============================================================================

List of generated methods in project: FirGeneration

THIS TEXT DESCRIPTION IS GENERATED BY THE TOOL. DO NOT MODIFY IT.

=============================================================================

Module "DFR1"      (bean DSP_Func_DFR)

- DFR1_dfr16FIR -The method computes a Finite Impulse Response, (FIR), filter for a vector of fractional data values.

Prior to any call to fir, the FIR filter data structure referred to by the first parameter pFIR must be

allocated and initialized. This allocation and initialization can be done either statically by declaring a

dfr16_tFirStruct data structure and then initializing it with a call to dfr16FIRInit, or dynamically by calling

dfr16FIRCreate.

- DFR1_dfr16FIRCreate -The method performs memory allocation and initialization for the fir filter function.

- DFR1_dfr16FIRHistory -The method reinitializes the Finite Impulse Response, (FIR), filter history buffer data values.

Prior to any call to firHistory, the FIR filter must be initialized via a call to firCreate. The firHistory

function uses the private data structure established by firCreate to establish the past history of data

elements required by the FIR filter computation. The argument pX must point to n fractional data elements,

where n is the number of coefficients used to establish the FIR filter in the call to firCreate.

- DFR1_dfr16FIRDestroy -The method deallocates the data structure(s) initially allocated by firCreate.

- DFR1_dfr16FIRInit -The method initializes the fir filter function from a previously allocated dfr16_tFirStruct data

structure. Typically, this dfr16_tFirStruct data structure is allocated statically by the caller prior to the

firInit call. However, firCreate also uses firInit to initialize the dfr16_tFirStruct that it dynamically

allocates.

- DFR1_dfr16FIRs -The method computes a Finite Impulse Response (FIR) filter for a single fractional data value.   Prior to

any call to fir, the FIR filter data structure referred to by the first parameter pFIR must be allocated and

initialized. This allocation and initialization can be done either statically by declaring a dfr16_tFirStruct

data structure and then initializing it with a call to dfr16FIRInit, or dynamically by calling dfr16FIRCreate.

- DFR1_dfr16FIRInt -The method computes an interpolating Finite Impulse Response, (FIR), filter for a vector of fractional

data values.   Prior to any call to firint, the FIR filter data structure referred to by the first parameter

pFIRINT must be allocated and initialized. This allocation and initialization can be done either statically by

declaring a dfr16_tFirIntStruct data structure and then initializing it with a call to dfr16FIRIntInit, or

dynamically by calling dfr16FIRIntCreate.

- DFR1_dfr16FIRIntCreate -The method performs the initialization for the firint filter function. The firintCreate function

allocates and initializes a data structure, dfr16_tFirIntStruct, which is used by firint to preserve the

filters state between calls. The data structure preserves the pC pointer to the vector of interpolating FIR

filter coefficients. The function firintCreate also allocates a buffer to save the past history of n data

elements required by the interpolating FIR filter computation; a pointer to this buffer is stored in the data

structure returned by firint. So that firint can achieve maximum efficiency using modulo addressing,

firintCreate allocates the history buffer from the system heap such that its address starts on a k-bit boundary,

where k=log2(n).

- DFR1_dfr16FIRIntDestroy -The method deallocates the data structure initially allocated by firintCreate.

- DFR1_dfr16FIRIntInit -The method initializes the firint filter function from a previously allocated dfr16_tFirIntStruct

data structure. Typically, this dfr16_tFirIntStruct data structure is allocated statically by the caller prior

to the firintInit call. However, firintCreate also uses firintInit to initialize the dfr16_tFirIntStruct that

it dynamically allocates.

- DFR1_dfr16FIRDec -The method computes a decimating Finite Impulse Response, (FIR), filter for a vector of fractional data

values.   Prior to any call to firdec, the FIR filter data structure referred to by the first parameter pFIRDEC

must be allocated and initialized. This allocation and initialization can be done either statically by

declaring a dfr16_tFirDecStruct data structure and then initializing it with a call to dfr16FIRDecInit, or

dynamically by calling dfr16FIRDecCreate.

- DFR1_dfr16FIRDecCreate -The method performs the initialization for the firdec filter function. The function firdecCreate

allocates and initializes a data structure, dfr16_tFirDecStruct, which is used by firdec to preserve the

filters state between calls. The data structure preserves the pC pointer to the vector of decimating FIR

filter coefficients. The firdecCreate function also allocates a buffer to save the past history of n data

elements required by the decimating FIR filter computation; a pointer to this buffer is stored in the data

structure pointed to by pFIRDEC. So firdec can achieve maximum efficiency using modulo addressing, firdecCreate

allocates the history buffer from the system heap such that its address starts on a k-bit boundary, where

k=log2(n).

- DFR1_dfr16FIRDecDestroy -The method deallocates the data structure initially allocated by firdecCreate.

- DFR1_dfr16FIRDecInit -The method initializes the firdec filter function from a previously allocated dfr16_tFirDecStruct

data structure. Typically, this dfr16_tFirDecStruct data structure is allocated statically by the caller prior

to the firdecInit call. However, firdecCreate also uses firdecInit to initialize the dfr16_tFirDecStruct that

it dynamically allocates.

- DFR1_dfr16IIR -The method computes an Infinite Impulse Response, (IIR), filter for a vector of fractional data values

using a cascade filter of biquad coefficients.   Prior to any call to iir, the IIR filter data structure

referred to by the first parameter pIIR must be allocated and initialized. This allocation and initialization

can be done either statically by declaring a dfr16_tIirStruct data structure and then initializing it with a

call to dfr16IIRInit, or dynamically by calling dfr16IIRCreate.

- DFR1_dfr16IIRCreate -The method performs the initialization for the cascade iir filter function. The function iirCreate

allocates and initializes a data structure, which is used by iir to preserve the filters state between calls.

The data structure preserves the pC pointer to the linearized vector of n biquad IIR filter coefficients. The

iirCreate function also allocates a buffer to save the past history of two data elements (as shown in the

formula below using variable w) per biquad required by the cascade IIR filter computation; a pointer to this

buffer is stored in the private data structure. So that iir can achieve maximum efficiency using modulo

addressing, iirCreate allocates the history buffer from the system heap such that its address starts on a k-bit

boundary, where: k = log(2)(2 * n).

- DFR1_dfr16IIRDestroy -The method deallocates the data structure(s) initially allocated by iirCreate.

- DFR1_dfr16IIRInit -The method performs the initialization for the cascade iir filter function and initializes the function

from a previously-allocated dfr16_tIirStruct data structure. Typically, the dfr16_tIirStruct data structure is

allocated statically by the caller prior to the iirInit call. The iirCreate function also uses iirInit to

initialize the dfr16_tIirStruct that it dynamically allocates.

- DFR1_dfr16AutoCorr -The method computes the first nz points of auto-correlation of a vector of fractional data values.

- DFR1_dfr16Corr -The method computes the full length correlation of two vectors of fractional data values.

- DFR1_dfr16CFFT -The method computes an in place, radix-2, complex, decimation-in-time, (DIT), forward Fast Fourier

Transform function for a vector of complex fractional data values.   Prior to any call to cfft, the CFFT data

structure referred to by the first parameter pCFFT must be allocated and initialized. This allocation and

initialization can be done either statically by declaring a dfr16_tCFFTStruct data structure and then

initializing it with a call to dfr16CFFTInit, or dynamically by calling dfr16CFFTCreate.

- DFR1_dfr16CFFTCreate -The method performs the initialization for an in place, radix-2, complex, decimation-in-time (DIT),

forward Fast Fourier Transform (FFT) function, cfft, using complex fractional data values. The function

cfftCreate allocates and initializes a data structure, which is used by cfft to preserve the FFT functions

state between calls.

- DFR1_dfr16CFFTDestroy -The method deallocates the data structure(s) initially allocated by cfftCreate.

- DFR1_dfr16CFFTInit -The method performs the initialization for an in place, radix-2, complex, decimation-in-time (DIT),

forward Fast Fourier Transform (FFT) function, cfft, using complex fractional data values. Initializes the

function from a previously-allocated dfr16_tCFFTStruct data structure. Typically, the dfr16_tCFFTStruct data

structure is allocated statically by the caller prior to the cfftInit call. The cfftCreate function also uses

cfftInit to initialize the dfr16_tCFFTStruct that it dynamically allocates.

- DFR1_dfr16CIFFT -The method computes an in place, radix-2, complex, decimation-in-time (DIT), inverse Fast Fourier

Transform function for a vector of complex fractional data values.   Prior to any call to cifft, the CFFT data

structure referred to by the first parameter pCIFFT must be allocated and initialized. This allocation and

initialization can be done either statically by declaring a dfr16_tCFFTStruct data structure and then

initializing it with a call to dfr16CIFFTInit, or dynamically by calling dfr16CIFFTCreate.

- DFR1_dfr16CIFFTCreate -The method performs the initialization for an in place, radix-2, complex, decimation-in-time (DIT),

inverse Fast Fourier Transform, (IFFT), using complex fractional data values. The function cifftCreate

allocates and initializes a data structure, which is used by cifft to preserve the IFFT functions state

between calls.

- DFR1_dfr16CIFFTDestroy -The method deallocates the data structure (s) initially allocated by cifftCreate.

- DFR1_dfr16CIFFTInit -The method performs the initialization for an in place, radix-2, complex, decimation-in-time (DIT),

inverse Fast Fourier Transform, (IFFT), using complex fractional data values. Initializes the function from a

previously-allocated dfr16_tCFFTStruct data structure. Typically, the dfr16_tCFFTStruct data structure is

allocated statically by the caller prior to the cifftInit call. The cifftCreate function also uses cifftInit to

initialize allocate the dfr16_tCFFTStruct dynamically.

- DFR1_dfr16RFFT -The method computes an in place, radix-2, real, decimation-in-time (DIT) forward Fast Fourier Transform

function for a vector of fractional data values.   Prior to any call to rfft, the RFFT data structure referred

to by the first parameter pRFFT must be allocated and initialized. This allocation and initialization can be

done either statically by declaring a dfr16_tRFFTStruct data structure and then initializing it with a call to

dfr16RFFTInit, or dynamically by calling dfr16RFFTCreate.

- DFR1_dfr16RFFTCreate -The method performs the initialization for an in place, radix-2, real, decimation-in-time, (DIT),

forward Fast Fourier Transform, (FFT), function using fractional data values. The function rfftCreate allocates

and initializes a data structure, which is used by rfft to preserve the FFT functions state between calls.

- DFR1_dfr16RFFTDestroy -The method deallocates the data structure(s) initially allocated by rfftCreate.

- DFR1_dfr16RFFTInit -The method performs the initialization for an in place, radix-2, real, decimation-in-time (DIT),

forward Fast Fourier Transform (FFT) function, rfft, using complex fractional data values. Initializes the

function from a previously allocated dfr16_tRFFTStruct data structure. Typically, the dfr16_tRFFTStruct data

structure is allocated statically by the caller prior to the rfftInit call. The rfftCreate function also uses

rfftInit to initialize the dfr16_tRFFTStruct that it dynamically allocates.

- DFR1_dfr16RIFFT -The method computes an in place, radix-2, decimation-in-time, (DIT), inverse Fast Fourier Transform

function for a structure of complex data values.   Prior to any call to rifft, the RFFT data structure referred

to by the first parameter pRIFFT must be allocated and initialized. This allocation and initialization can be

done either statically by declaring a dfr16_tRFFTStruct data structure and then initializing it with a call to

dfr16RIFFTInit, or dynamically by calling dfr16RIFFTCreate.

- DFR1_dfr16RIFFTCreate -The method performs the initialization for an in place, inverse real Fast Fourier Transform, (IFFT),

function using fractional data values. The function rifftCreate allocates and initializes a data structure,

which is used by rifft to preserve the FFT functions state between calls.

- DFR1_dfr16RIFFTDestroy -The method deallocates the data structure(s) initially allocated by rifftCreate.

- DFR1_dfr16RIFFTInit -The method performs the initialization for an in place, radix-2, real, decimation-in-time (DIT),

forward Fast Fourier Transform (FFT) function, rifft, using complex fractional data values. Initializes the

function from a previously allocated dfr16_tRIFFTStruct data structure. Typically, the dfr16_tRIFFTStruct data

structure is allocated statically by the caller prior to the rifftInit call. The rifftCreate function also uses

rifftInit to initialize the dfr16_tRIFFTStruct that it dynamically allocates.

- DFR1_dfr16Cbitrev -The method bit-reverses the position of n complex data elements in the input vector, pointed to by pX.

The cbitrev function is used to convert normal, (linear), order complex vectors to bit-reversed complex vectors,

and vice-versa, for use in Fast Fourier Transform functions.

Module "MEM1"      (bean DSP_MEM)

- MEM1_memCallocEM -The memCallocEM function dynamically allocates an array with elements initialized to zero. The

memCallocEM function first tries to reallocate the memory from the external memory partition. For example see

<Typical Usage > page.

- MEM1_memCallocIM -The memCallocIM function dynamically allocates an array with elements initialized to zero. The

memCallocIM function first tries to reallocate the memory from the internal memory partition. For example see

<Typical Usage > page.

- MEM1_memFreeEM -The memFreeEM function deallocates a memory block in external memory that previously had been dynamically

allocated with the routine memMallocEM. For example see <Typical Usage > page.

- MEM1_memFreeIM -The memFreeIM function deallocates a memory block in internal memory that previously had been dynamically

allocated with the routine memMallocIM. For example see <Typical Usage > page.

- MEM1_memIsAligned -The memIsAligned function checks the address of a memory block to determine whether it is properly

aligned to use modulo addressing. For example see <Typical Usage > page.

- MEM1_memIsEM -The memIsEM function checks the address of a memory block to determine whether it is in internal memory or

external memory. For example see <Typical Usage > page.

- MEM1_memIsIM -The memIsIM function checks the address of a memory block to determine whether it is in internal memory or

external memory. For example see <Typical Usage > page.

- MEM1_memMallocAlignedEM -The memMallocAlignedEM function allocates dynamic memory of the specified size from the memory

partitions and aligns the memory to use modulo addressing. The function memMallocAlignedEM first tries to

allocate the memory, properly aligned, from the external memory partition. However, if space is insufficient in

the external memory partition to satisfy the dynamic memory allocation, or if the memory request cannot be

properly aligned, memMallocAlignedEM then tries the allocation from the internal memory partition. If

memMallocAlignedEM cannot allocate an aligned buffer from either partition, it calls memMallocEM to try to

allocate a buffer of the correct size without the proper alignment. For example see <Typical Usage > page.

- MEM1_memMallocAlignedIM -The memMallocAlignedIM function allocates dynamic memory of the specified size from the memory

partitions and aligns the memory properly to use modulo addressing. The function memMallocAlignedIM first tries

to allocate the memory, properly aligned, from the internal memory partition. However, if not enough space

exists in the internal memory partition to satisfy the dynamic memory allocation, or if the memory request

cannot be properly aligned, memMallocAlignedIM then tries the allocation from the external memory partition. If

memMallocAlignedIM cannot allocate an aligned buffer from either partition, it calls memMallocIM to try to

allocate a buffer of the correct size without the proper alignment. For example see <Typical Usage > page.

- MEM1_memMallocEM -The memMallocEM function allocates dynamic memory of the specified size from the memory partitions. The

function memMallocEM first tries to allocate the memory from the external memory partition. However, if not

enough space exists in the external memory partition to satisfy the dynamic memory allocation, memMallocEM then

tries the allocation from the internal memory partition. For example see <Typical Usage > page.

- MEM1_memMallocIM -The memMallocIM function allocates dynamic memory of the specified size from the memory partitions. The

function memMallocIM first tries to allocate the memory from the internal memory partition. However, if not

enough space exists in the internal memory partition to satisfy the dynamic memory allocation, memMallocIM then

tries the allocation from the external memory partition. For example see <Typical Usage > page.

- MEM1_memReallocEM -The memReallocEM function dynamically reallocates and resizes a memory buffer to the specified size

from the memory partitions. The function memReallocEM first tries to reallocate the memory from the external

memory partition. However, if not enough space exists in the external memory partition to satisfy the dynamic

memory allocation, memReallocEM then tries the allocation from the internal memory partition. For example see

<Typical Usage > page.

- MEM1_memReallocIM -The memReallocIM function dynamically reallocates and resizes a memory buffer to the specified size

from the memory partitions. The function memReallocIM first tries to reallocate the memory from the internal

memory partition. However, if not enough space exists in the internal memory partition to satisfy the dynamic

memory allocation, memReallocIM then tries the allocation from the external memory partition For example see

<Typical Usage > page.

- MEM1_memMemcpy -The memcpy routine copies characters between memory buffers For example see <Typical Usage > page.

- MEM1_memMemset -The memset function sets user specified number of bytes in memory to a specific character For example see

<Typical Usage > page.

- MEM1_memMemsetP -The memset function sets user specified number of bytes in 'P' memory to a specific character

- MEM1_memCopyPtoX -The memCopyPtoX copies a block of data from the P (program) data memory to the X data memory. The

memCopyPtoX executes the specific instructions required to read data from program P data memory, as opposed to

X data. For example see <Typical Usage > page.

- MEM1_memCopyXtoP -The memCopyXtoP copies a block of data from the X data memory to the P (program) data memory. The

memCopyXtoP executes the specific instructions required to write data to program P data memory, as opposed to X

data. For example see <Typical Usage > page.

- MEM1_memCopyPtoP -The memCopyPtoP copies a block of data from the P (program) data memory to the P data memory. The

memCopyPtoP executes the specific instructions required to read data from program P data memory.

- MEM1_memReadP16 -The function memReadP16 reads a 16-bit word from the program data (P data), address space. The memReadP16

executes the specific instructions to reference program P data, as opposed to X data. For example see <Typical

Usage > page.

- MEM1_memReadP32 -The function memReadP32 reads a 32-bit word from the program data (P data) address space. The memReadP32

executes the specific instructions to reference program P data, as opposed to X data. For example see <Typical

Usage > page.

- MEM1_memWriteP16 -The memWriteP16 function writes a 16-bit value into the program data, (P data), address space. The

memWriteP16 executes the specific instructions required to write to program (P) data, as opposed to X data. For

example see <Typical Usage > page.

- MEM1_memWriteP32 -The memWriteP32 function writes a 32-bit value into the program data, (P data), address space. The

memWriteP32 executes specific instructions required to write to program P data, as opposed to X data. For

example see <Typical Usage > page.

Module "Cpu"       (bean 56F8006_48_LQFP)

- Cpu_EnableInt -Enables all maskable interrupts

- Cpu_DisableInt -Disables all maskable interrupts

- Cpu_SetWaitMode -Sets low power mode - Wait mode.  For more information about the wait mode see this CPU documentation.

Release from wait mode: Reset or interrupt

- Cpu_SetStopMode -Sets low power mode - Stop mode.  For more information about the stop mode see this CPU documentation.

===================================================================================