=============================================================================

List of generated methods in project: voltmeter_led

THIS TEXT DESCRIPTION IS GENERATED BY THE TOOL. DO NOT MODIFY IT.

=============================================================================

Module "AD1"       (bean ADC)

- AD1_Enable -Enables A/D converter bean. <Events> may be generated (<DisableEvent>/<EnableEvent>). If possible, this method

switches on A/D converter device, voltage reference, etc.

- AD1_Disable -Disables A/D converter bean. No <events> will be generated. If possible, this method switches off A/D

converter device, voltage reference, etc.

- AD1_Start -This method starts continuous conversion on all channels that are set in the bean inspector. When each

measurement on all channels has finished the <OnEnd > event may be invoked. This method is not available if the

<interrupt service> is disabled and the device doesn't support the continuous mode. Note: If time of

measurement is too short and the instruction clock is too slow then the conversion complete interrupt and its

handler may cause a system overflow.

- AD1_Stop -This method stops the continuous measurement or disables a trigger mode (if supported by HW), which has been

started by one of the following methods:  Version specific information for Freescale HCS08 and HC08 derivatives

]     - <Start>     - <EnableIntChanTrigger>    - <EnableExtChanTrigger> Version specific information for

Freescale HCS12 and HCS12X derivatives ]     - <Start>    - <EnableExtTrigger>    - <EnableExtChanTrigger>

- <EnableIntTrigger>    - <EnableIntChanTrigger> Version specific information for Freescale 56800 derivatives ]

- <Start>     - <EnableIntTrigger>    - <EnableIntChanTrigger> Version specific information for Freescale MCF

derivatives ]     - <Start>     - <EnableExtTrigger>    - <EnableExtChanTrigger> Version specific information

not for Freescale 56800 and HCS08 and HC08 and HCS12 and HCS12X derivatives ]     - <Start>     -

EnableInt(Chan)Trigger     - EnableExt(Chan)Trigger The Stop method is available if one of the previously

mentioned methods is supported by A/D converter device and is enabled to be generated.

- AD1_Measure -This method performs one measurement on all channels that are set in the bean inspector. (Note: If the

<number of conversions> is more than one the conversion of A/D channels is performed specified number of times.)

- AD1_GetChanValue -Returns the last measured value of the required channel. Format and width of the value is a native

format of the A/D converter.

- AD1_GetValue16 -This method returns the last measured values of all channels justified to the left. Compared with

<GetValue> method this method returns more accurate result if the <number of conversions> is greater than 1 and

<AD resolution> is less than 16 bits. In addition, the user code dependency on <AD resolution> is eliminated.

Module "PGA2"      (bean Init_PGA_56800)

- PGA2_Init -This method initializes registers of the PGA module according to this Peripheral Initialization Bean settings.

Call this method in the user code to initialize the module. By default, the method is called by PE

automatically; see <Call Init method> property of the bean for more details.

Module "PWMC1"     (bean PWMMC)

- PWMC1_SetDuty -Setting duty(value) register of selected channel.  The value is loaded after calling Load() method.

- PWMC1_Load -Apply last setting of the methods SetDuty, SetDutyPercent, SetPeriod and SetPrescaler.

- PWMC1_OutputPadEnable -Method enables output pads.

- PWMC1_OutputPadDisable -Method disables output pads.

Module "PDB1"      (bean Init_PDB_56800)

- PDB1_Init -This method initializes registers of the PDB module according to this Peripheral Initialization Bean settings.

Call this method in the user code to initialize the module. By default, the method is called by PE

automatically; see <Call Init method> property of the bean for more details.

Module "Timer0"    (bean Init_TMR_56800)

- Timer0_Init -This method initializes registers of the TMR module according to this Peripheral Initialization Bean settings.

Call this method in the user code to initialize the module. By default, the method is called by PE

automatically; see "Call Init method" property of the bean for more details.

Module "PC_M1"     (bean PC_Master)

Module "Cpu"       (bean 56F8006_48_LQFP)

- Cpu_EnableInt -Enables all maskable interrupts

- Cpu_DisableInt -Disables all maskable interrupts

- Cpu_SetWaitMode -Sets low power mode - Wait mode.  For more information about the wait mode see this CPU documentation.

Release from wait mode: Reset or interrupt

- Cpu_SetStopMode -Sets low power mode - Stop mode.  For more information about the stop mode see this CPU documentation.

- Cpu_Delay100US -This method realizes software delay. The length of delay is at least 100 microsecond multiply input

parameter [us100]. As the delay implementation is not based on real clock, the delay time may be increased by

interrupt service routines processed during the delay. Adding read wait states for external program memory can

cause delay extension as well. The method is independent on selected speed mode.

===================================================================================